Finishing Table

Hazards:

| Hazards | Controls |
| --- | --- |
| Noise | Class 5 hearing protection must be worn. These may be ear plugs or ear muffs. |
| Manual handling | Use appropriate lifting techniques: bend your knees, not your back. Do not twist. Turn using your feet and keeping your body straight. Hold timber close to your body. Gloves are provided and are optional. |
| Nail gun operation | Only trained and authorised people are to operate the nail gun. The operator’s hands must be away from the firing area. Nail the bottom of the timber first. |
| Trip hazards | Be aware of where the air hose is. Keep it out of the way. |

PPE:

Qualification:

Only trained and authorised staff are to work at the finishing table.

Pre-start checks:

Check the condition of the nail gun. Ensure the hoses are in good condition and the coupling is secure.

Operation: Finishing Table

|  | Step | Action |
| --- | --- | --- |
|  | 1 | - Add lintels, etc. as required. - Rip down where needed to correct size. - Shoot skew nails in from nogs to stud. |

| Step | Action |
| --- | --- |
| 2 | - Check the frames to ensure there is no bowing. - If there is bowing, remove the stud, adjust the nog and replace the stud. - Apply additional items such as angle braces, etc. - Check overall quality and ensure it is within the customer’s expectation: all components flush, all nails nailed tidily, all timber aligned. - Nail frames to each other, on outside of frame, for transport stability. |

| Step | Action |
| --- | --- |
| 3 | - Move the frame to the stack for removal. - Use two people if the frame is heavy. - Strap finished product with metal band strapping. - Wrap the finished product with plastic sheeting and nail into place. - Apply the correct label to the finished packet. - Check the measurements of the finished product against the label. - Move outside for removal by the forklift operator. |

Shut Down:

Tidy up the area.

| DATE | TRAINEE NAME | SIGNATURE | TRAINER | SIGNATURE |
| --- | --- | --- | --- | --- |